# Software Requirements Specification (SRS)

Name: CrowdSenseNet

Design and Implementation of a Crowdsensed Drive Test Mobile Application for Coverage Prediction

## 5.1 Introduction

### 5.1.1 Purpose

This document provides the complete Software Requirements Specification (SRS) . The purpose of this document is to specify the requirements for CrowdSenseNet, a system designed to address the lack of transparent network coverage data in Cameroon. By leveraging crowdsensing technology, the system aims to empower both consumers and network operators with real-time, accurate signal maps, replacing expensive traditional drive-tests with a community-driven mobile solution.”

### 5.1.2 Document Conventions

- Must Have: Critical requirements for the Minimum Viable Product (MVP)

- Should Have: Important features that add significant value but are not blocking

- Signal measurements are expressed in dBm for power and dB for quality

### 5.1.3 Intended Audience

- Developers and architects: focus on Sections 5.3 (System Features) and 5.7 (Architecture)

- Network engineers: focus on Section 5.1.5 (Definitions) and Section 5.6 (Data Requirements)

- Academic supervisors: focus on Section 6 (Stakeholder Validation) and the traceability matrix at the end

### 5.1.4 Project Scope

CrowdSenseNet is an Android application that turns ordinary smartphones into Class-2 IoT sensing devices. It passively collects cellular metrics (RSRP, RSRQ, SINR) to build real-time coverage heatmaps and predict coverage holes (RSRP below -110 dBm) using a machine learning model. iOS support is excluded in this phase due to API restrictions and scope limitations.

The Minimum Viable Product (MVP) covers: signal data collection (RSRP, RSRQ, SINR, GPS), local offline storage, backend data upload, a basic coverage map, and ML-based coverage hole prediction.

### 5.1.5 Key Definitions

| Term | Definition |
| --- | --- |
| RSRP | Reference Signal Received Power. The primary LTE signal strength metric. Coverage hole threshold: below -110 dBm |
| RSRQ | Reference Signal Received Quality. Measures signal quality relative to interference |
| SINR | Signal to Interference-plus-Noise Ratio. Indicates signal clarity |
| Coverage Hole | Any location where predicted RSRP is below -110 dBm |
| UUID | Universally Unique Identifier. A random anonymous token generated on first launch |
| Crowdsensing | Collecting data passively from many users to build a picture of real-world conditions |
| Drive Test | Traditional method of measuring mobile coverage using engineers with specialised equipment in a vehicle |

Table 5.1: Key definitions and terminology

## 5.2 Overall Description

### 5.2.1 Product Perspective

CrowdSenseNet is a standalone Android application operating within a three-tier architecture:

- Tier 1 (Client): Android app written in Kotlin for data collection and local SQLite storage

- Tier 2 (Backend): Python FastAPI server for data validation and PostgreSQL storage

- Tier 3 (Analytics): Python ML model for coverage prediction and heatmap generation

### 5.2.2 User Classes

- General Contributors: Non-technical users who passively contribute signal data.

- Network Engineers: High-technical users at MTN/Orange who access maps to fix problem areas.

- System Administrators: Experts managing the backend, database, and ML updates.

### 5.2.3 Operating Environment

- Mobile platform: Android 8.0 (API level 26) minimum

- Local database: Room (SQLite-based)

- Remote database: PostgreSQL with PostGIS for spatial data

- Network: 4G, 5G, or WiFi for data synchronisation

### 5.2.4 Constraints

- Battery constraint: Adaptive sampling is required to prevent unacceptable drain

- Geographic scope: Limited to the Cameroon bounding box (Latitude 1.65N to 13.08N)

- Permission dependency: Requires ACCESS_FINE_LOCATION and READ_PHONE_STATE permissions

### 5.2.5 Assumptions and Dependencies

- Users must grant required hardware permissions on first launch

- A minimum of 50 readings per 1 km squared is required before the ML model can generate a prediction

- Android TelephonyManager API must support SINR on the user's device (varies by Android version and chipset)

## 5.3 System Features

### 5.3.1 Functional Requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-01 | Single-tap session start/stop from the Home screen | Must Have |
| FR-02 | Auto-pause collection when the device is stationary for more than 30 seconds | Must Have |
| FR-03 | Auto-resume collection when device speed exceeds 5 km/h | Must Have |
| FR-04 | Capture RSRP, RSRQ, SINR, and RSSI via TelephonyManager API | Must Have |
| FR-05 | Capture GPS coordinates with every signal reading | Must Have |
| FR-06 | Discard readings where GPS accuracy is worse than 15 metres | Must Have |
| FR-07 | Store all readings immediately in local Room database | Must Have |
| FR-08 | Generate and store a random anonymous UUID on first launch | Must Have |
| FR-09 | Upload data to server over WiFi (default) | Should Have |
| FR-10 | Allow user to enable mobile data upload via Settings toggle | Should Have |
| FR-11 | Retry failed uploads automatically every 30 minutes | Should Have |
| FR-12 | Export collected data in GeoJSON and CSV formats | Should Have |
| FR-13 | Render a coverage heatmap on the Map screen | Should Have |
| FR-14 | Display a real-time signal dashboard on the Home screen | Could Have |
| FR-15 | Allow user to adjust sampling interval (5 to 30 seconds) in Settings | Could Have |
| FR-16 | Tag readings as indoor or outdoor based on sensor data | Could Have |

Table 5.3: Functional requirements with IDs and priorities

## 5.4 External Interface Requirements

### 5.4.1 User Interface

- Modern Design: Built with Material Design 3 and responsive ConstraintLayouts

- Four core screens: Home (Dashboard), Map (Heatmap), History, and Settings

- All key actions must be reachable within 3 taps

- Supports English and French based on device language settings

### 5.4.2 Hardware Interfaces

- GPS: FusedLocationProvider API for battery-efficient location tracking

- Cellular modem: TelephonyManager for hardware-level signal parameter access

### 5.4.3 Software Interfaces

- Maps: Google Maps API or OpenStreetMap for the heatmap layer

- Server API: REST over HTTPS for all app-to-server communication

- Local DB: Room (Android) linked to SQLite for offline storage

### 5.4.4 Communication Interfaces

- All transmissions must use TLS 1.2 or higher

- WiFi-only upload is the default; mobile data upload is opt-in via Settings

## 5.5 Non-Functional Requirements

### 5.5.1 Performance

- App must launch and reach the Home screen within 3 seconds

- Heatmap must render within 4 seconds on a stable connection

### 5.5.2 Resource Constraints

- Background collection must consume less than 5% battery per hour

- Local storage is capped at 50MB with automatic deletion of synced records

### 5.5.3 Security

- The app must not collect name, email, IMEI, or IP address

- Backend API endpoints must require authentication tokens

- All data transmissions must be encrypted using TLS 1.2 or higher

### 5.5.4 Reliability

- No data loss during app crashes: all readings are written to local DB immediately

- Failed uploads are retried automatically every 30 minutes

### 5.5.5 Usability

- English and French language support based on device settings

- No more than 3 steps to enable or disable data collection

### 5.5.6 ML Accuracy

- The ML model must achieve at least 80% classification accuracy for coverage hole prediction

- Predictions require a minimum of 50 readings within 1 km squared to be valid